Knowledge

- Sample precast drawings

D:\QA Agent\qa_agent\QA AI Drawing\QA Knowledge\Sample drawings

- explain terminology

- Naming:

- Ansicht: View / Elevation of formwork in technical drawings.

- Schnitt: Section / Cross-section in technical drawings. Symbol of section is like this:

- Bewehrung: Reinforcement / Rebar in construction drawings

- Draufsicht = Top view / Plan view in technical drawings

- Stabliste = Bar list / Rebar schedule in reinforcement drawings.

- Einbauteilliste = Embedded parts list/built in part list. This is the table summarize the product need to place inside the precast element

- Montageteilliste (je Fertigteil) = Assembly parts list (This is the table summarize mounting part for each precast element).

- Wandansicht = Wall elevation / Wall view.

- Lastausgleichsgehänge = Load-balancing hanger / Suspension support.

- Title block = The information panel on a technical drawing containing project details such as drawing title, number, scale, date, revision, and author. Spell check node

- Mattenstahlliste is a table listing of mesh rebar

- Matten-Schneideskizze = A drawing/sketch showing how reinforcement mesh is cut and shaped before installation.

- Spell check node

will be check all of this issues below and report this pass or fail if fail display why it’s fail.

- Check whether all text in the PDF is spelled correctly.

- Check whether the section names are correct.
For example, if sections 1-1, 2-2, and 3-3 are shown in the Ansicht or Bewehrung, the corresponding sections Schnitt 1-1, Schnitt 2-2, and Schnitt 3-3 must all be provided.

- Check whether the component name on the Wandansicht matches the drawing name in the title block.

- Check whether the drawing scales match the title block.
If any scale in the drawing (section, detail…) differs from the title block, report it as FAIL and update the title block accordingly.

- Check whether the wall formwork Schnitt grid lines match the Wandansicht.

- Check that the Built-in Parts List and Mounting Parts List are provided.

- Check whether the quantities of built-in parts and mounting parts in the Schnitt and Ansicht match the quantities in the schedules.

The second presentation format: For the second presentation format, only check whether all built-in parts have labels.
For example, if there are 4 built-in parts in the drawing, then verify that all 4 labels number are shown in the section view.

- Check that the wall has a 3D view included.
The wall view must match the elevation/section view (“Ansicht”).

Output format – Spell check node

Each check result is reported as:

[CHECK NAME]: PASS | FAIL

If FAIL → Reason: <what was found> vs. <what was expected>

Summary line at the end:

Spell Check Summary: X checks total, Y passed, Z failed.

Overall status: PASS (all checks passed) | FAIL (one or more checks failed)

- Bend check node

will be check all of this issues below and report this pass or fail if fail display why it’s fail.

- Check whether the rebar positions on the drawing (excluding rebar positions greater than 100) match the positions in the detail drawings.

- check the rebar position of the mesh reinforcement in the drawing, if available.

- check that the ratio of used mesh reinforcement to the total reinforcement quantity is greater than 85%.

Ratio = (226.09/231.99)*100 = 97.46% Pass

- Check whether the total mass in the Stabliste is correct.

Also Check mass for Mattenstahlliste if available

- Check the Bewehrung sections and their sub-sections, and count the created rebar schemas.
Each rebar type in the Stabliste must be represented by at least one schema.

Example: in the Stabliste contains positions 1–19 and 100–104, each of these rebar positions must have at least one corresponding schema.

- Check each rebar schema to verify that the bending angles is correct.

- Ø8 bending factor is 4 → dbr = 3.2

- Ø10 bending factor is 4 → dbr = 4

- Ø12 bending factor is 4 → dbr = 4.8

- Ø16 bending factor is 4 → dbr = 6.4

- Ø20 bending factor is 7 → dbr = 14

- Ø24 bending factor is 7 → dbr = 16.8

- Ø28 bending factor is 7 → dbr = 19.6

- Check each rebar schema to verify that the total length L is correct and match with Einzel Lange in Stabliste

Example:

Output format – Bend check node

Each check result is reported as:

[CHECK NAME]: PASS | FAIL

If FAIL → Position: <pos number>, Reason: <discrepancy description>

For ratio checks: Calculated ratio = X %, Threshold = 85 % → PASS / FAIL

Summary line at the end:

Bend Check Summary: X checks total, Y passed, Z failed.

Overall status: PASS (all checks passed) | FAIL (one or more checks failed)

- Rebar check node

will be check all of this issues below and report this pass or fail if fail display why it’s fail.

- Check that the Spacers/Clamps reinforcement label includes the suffix “-M.E.” at the end.

- check the width of the horizontal and vertical pin reinforcement.

- Vertical pin = wall_width – 2*Cv

Cv value in detail

Width_ver_pin = 30 – 2*2.5 = 25

- Horizontal pin = wall_width – 2*Cv – 2*Layer1 rebar diameter (round down)

Cv value in detail

Layer1 rebar diameter detect the label in the side section of the wall (example is position 12)

Width_hor_pin = 30 – 2*2.5 – 2*1.2 = 22.6 ~ 22

- check the width of the Spacers/Clamps reinforcement.

Spacers/Clamps = wall_width – 2*Cv – 2*Spacer diameter (round up)

Cv value in detail

Layer1 rebar diameter detect the label in the side section of the wall (example is position 12)

Width_hor_pin = 30 – 2*2.5 + 2*0.8 = 26.6 ~ 27

Output format – Rebar check node

Each check result is reported as:

[CHECK NAME]: PASS | FAIL

If FAIL → Location: <view / position>, Formula used: <formula>, Calculated: <value>, Declared: <value>

Summary line at the end:

Rebar Check Summary: X checks total, Y passed, Z failed.

Overall status: PASS (all checks passed) | FAIL (one or more checks failed)